# GENERAL DESCRIPTION

  - The unit shall be a highly visible, outdoor-rated LED Blue Light, model ETP-EL, no substitutions, with built-in flash activation relay.

  - Unit shall have a low power mode consuming less than 2 watts.

  - Unit shall operate from -40ºF to 150ºF.

# Construction

  - Unit shall be housed in a vandal resistant, blue polycarbonate refractor housing measuring approximately 6.9" H x 5" Diameter (6.6” diameter base)

  - Gasket shall be made from UV-resistant EPDM foam rubber

  - Unit shall weigh approximately 4 lbs.

  - Refractor housing shall have a prismatic pattern to increase visibility at greater distances.

# Lighting

  - Unit shall have ultra-bright LEDs that produce a peak of 209 lumens (exceeds 1,000,000 candle power).

  - Unit shall flash approximately 78 times per minute.

  - Unit shall continue to produce 70% of the iniital lumens after 50,000 hours of operation

# ELectrical

  - Unit shall operate on 10-30VDC or 20-28VAC.  120VAC version is also available.

  - Unit shall have two 18GA pairs, approximately 15 feet in length: one for power and one for activation of the built-in relay.

  - Light shall consume 7.5 watts in normal mode or 1.5 watts in low power mode.

# Mounting

  - Unit shall use Mounting Bracket, model ETP-MBKT, to mount to 4" square box.

# Options

  - Low voltage version, model ETP-EL 12/24, shall accept 12VDC or 24VAC/DC. This version shall have a low-power mode.

# Compliance

  - Unit shall be Certified to UL Standard 1598.

# Warranty

  - Equipment shall be warrantied against any defects in material and workmanship, under normal use, for a period of two years from date of installation. In the event system is found by manufacturer to be defective within the warranty period, manufacturer shall repair and/or replace any defective parts, provided the equipment is returned to manufacturer.

# MANUFACTURER

  - The Manufacturer shall be Talk-A-Phone Co. (773) 539-1100, 7530 N. Natchez Ave, Niles, Illinois 60714-3804, www.talkaphone.com.  THERE ARE NO EQUIVALENTS.